F6 Strip IP rcfgx Selection

Full Brightness 5000 NITS rcfgx:

If you are running shows between 70-100% brightness, then you can adjust this directly from the VIP Drive 83R Nova menu, using the rcfgx that comes loaded from Chauvet when purchasing the F6 Strip IP. Due to the way that LED’s are driven in the F6 Strip IP, you will see little to no noticeable loss in grayscale or refresh rate if you don’t dim the driver below 70% brightness.

Reduced Brightness rcfgx Files:

If you are running shows below 70% brightness, and you adjust this directly from the Drive 83R Nova menu, using the rcfgx that comes loaded from Chauvet when purchasing the F6 Strip IP, you will start to see an incremental loss in both grayscale and refresh rate of the F6 Strip IP. This is due to the loss in available dimming levels being sent to the F6 Strip IP.

In order to operate at these lower intensities, while still maintaining optimal image quality, you need to load one of the available reduced brightness rcfgx files. These are available in multiple options for you to choose from, and are available for download from the Chauvet Video website.

Please note: when you load the reduced brightness rcfgx files, you should set the driver brightness to 100% to start. If you need to reduce the brightness, you may do so up to 70% for each of these reduced brightness rcfgx files without degrading the image quality.